July 2000    V1.01

＝＝＝＝＝   RDC-5000 USB Driver (Windows 2000)　README　　＝＝＝＝＝

Thank you for purchasing a Ricoh RDC series digital camera. This document will lead you through the process of installing the required USB driver on a computer with a Windows 2000 operating system.

Please familiarize yourself with the following information and the information contained in the operation manuals before installing this driver.

CONTENTS

- Operating Environment

- Driver installation procedure

- Driver uninstallation procedure

- Cautionary information

1. Operating Environment

Compatible camera: RDC-5000/5300

Operating System requirements: A pre-installed version of Windows 2000 running on a computer with a USB port equipped as standard, the operation of which is guaranteed by the manufacturer. (Operating systems that have been upgraded to Windows 2000 from another version of Windows are not supported.)

Memory requirements: 128MB (265MB or more recommended)

Connection cable: Please use only the cable that was included in the RDC-5000/5300 package.

==Important==

*This USB driver cannot be used with operating systems other than the one specified.

*Computers that have been upgraded from an operating system other than the one specified cannot be supported and operation is not guaranteed.

*Connection via retrofit USB ports (i.e., those that were added using an expansion feature like PCI bus) cannot be supported.

*Changes in the operating system or new service packs may cause incorrect operation.

*This driver may fail to operate correctly if used in conjunction with a USB hub or other USB devices.

*To handle large files, such as uncompressed RDC-5000/5300 files, it is recommended that you use this driver in environments with larger memory capacity.

2. Driver installation procedure

Prior to Setup

*Quit all running applications and save all necessary data.

*Disconnect any other USB devices and Plug & Play devices.

*Please save the downloaded file to your computer’s hard drive and decompress (unzip) it in preparation for the installation.

2-1. Installing the driver by Plug & Play

Notes:

*The ability to install this driver in Windows 2000 is limited to users who have administrator accounts. Users who cannot log in as administrators will not be able to install this driver.

*Please do not disconnect the USB cable or turn off the camera or the computer while the driver is being installed.

- Turn the computer on, start Windows.

- Connect the camera to the computer with the USB cable. (A direct connection to the USB port is recommended.)

- Turn the camera’s mode dial to PC and move the power switch to the ON position.

- The Found New Hardware wizard should appear when Windows recognizes the camera. When it does, click Next to begin the installation.

- The next screen will ask you what you want to do. Select “Search for a suitable driver for my device (recommended)”and click Next.

- The next window will ask where to search for the driver. Select “Specify a location” and click Next.

- Click Browse and browse to the location where the downloaded files are stored.

- Select the “Rdcumass.inf” file and click Open.

- Confirm that the path is correct, then click OK.

- Click Next to begin copying the files.

- Click Finish when the wizard notifies you that it has finished the installation.

- Windows will recognize new hardware once again and the Found New Hardware wizard will appear. Click Next to continue with the installation.

- Repeat steps 5 through 12 as they appear above. (When you come to step 8, select “Rdcumass.inf” again.)

- Click Finish when the wizard notifies you that it has finished the installation.

Note: In some cases, Windows will display a message indicating that it could not find a digital signature on your driver. If this message appears, simply click Yes to continue the installation.

2-2 Installing from Setup.exe

*Do not connect the RDC-5000/5300 to the computer until the installation is complete. If you have already connected the RDC-5000/5300 and turned it on, refer to the “Installing the driver by Plug & Play” instructions in section 2-1 above.

- Browse to the folder where the downloaded files are stored and locate the “Driver_setup” folder. Double-click the "Setup.exe" icon inside to launch the installer, which will install the driver.

- After the driver is installed, connect the RDC-5000/5300 to the computer via the USB cable. Turn the camera’s mode dial to PC and move the power switch to the ON position.

- When Windows recognizes the camera the "New Hardware" window will launch and begin to build a driver database. Please wait until it finishes. (The driver database will only be built the first time you connect the RDC-5000/5300 to your computer. It will not appear during subsequent connections.)

- Restart your computer after the driver database is finished.

3. Driver uninstallation procedure

Prior to uninstallation

*Exit any running applications and save any necessary data.

*Disconnect any other USB or Plug & Play devices.

*Do not connect the camera to the computer at this time.

*The procedure described hereafter works for Setup.exe version 1.0.3.0 or later. Please right-click on the Setup.exe icon to display its properties and check the version number before proceeding.

- Move or copy the Setup.exe icon to your Desktop. Right-click the icon and select Create Shortcut from the context menu.

- Right-click on the shortcut icon created in the step above and select Properties from the context menu. Select the Shortcut tab in the Properties window.

- At the very end of the path specified in the Link field, type “ /U/V” (without the quotation marks around it, but with a space before the first slash) after the final quotation mark in the path. Click OK.

- Double-click the shortcut icon to start the uninstallation process. Click OK at the prompt.

- When the uninstallation is complete, remove the shortcut icon from your Desktop and restart your computer.

4. Cautionary information

- This driver is only intended to import data from your camera’s memory. To initialize your camera’s memory, delete files, or perform other functions, please use the camera itself instead of the computer.

- A direct connection to your computer’s USB port is recommended. Connecting via a USB hub, particularly one that does not require its own power supply, is to be avoided.

- Please disconnect the camera prior to shutting down your computer.

- By all means avoid interruption of data transmission (by power loss, disconnection of the USB cable, or forcible shutdown), as both the camera and the computer could be damaged as a result.

- Whenever possible, use the AC adapter instead of batteries to power the camera, especially when transferring uncompressed files or motion pictures. Should power failure occur during transmission, it may adversely affect the camera, the computer, or both.

- The camera does not operate with power supplied from the USB port.

- If you make a mistake in the operation of this product that produces an malfunction, data loss could result.

- Understand that Ricoh disavows any responsibility for damages resulting from loss of data or damage caused to this or other devices as a result of mishandling, improper use, poor condition of equipment or any other reason.

- This document may not be copied, in whole or in part, for any purpose.

- This program is protected by copyright law and international treaties.

----------------------------------------------------------------------------

(C)　2000 RICOH COMPANY,LTD. All Rights Reserved.

Microsoft, Windows, and MS are registered trademarks of Microsoft Corporation in the United States of America and other countries.